LBC

import numpy as np

pb = []

col = int(input("Enter the Parity bits : "))      # n-k

row = int(input("Enter the Message bits : "))     # k

# -----------------------------

# Construct Parity Matrix P

# -----------------------------

for i in range(row):

p = list(map(int, input(f"Enter row {i+1} (space separated) : ").split()))

pb.append(p)

P = np.array(pb, dtype=int)

# -----------------------------

# Generator Matrix G = [P | I]

# -----------------------------

I_k = np.eye(row, dtype=int)

G = np.hstack((P, I_k))

print("\nGenerator Matrix (G):")

for r in G:

print(" ".join(map(str, r)))

# Code parameters

k = row

n = G.shape[1]

# -----------------------------

# Generate all possible messages

# -----------------------------

messages = np.array([[int(x) for x in format(i, f'0{k}b')] for i in range(2**k)])

# Generate codewords

codewords = np.mod(np.dot(messages, G), 2)

print("\nMessage   Codeword   Weight")

weights = []

for i in range(len(messages)):

wt = np.sum(codewords[i])

weights.append(wt)

print(" ".join(map(str, messages[i])), "   ",

" ".join(map(str, codewords[i])), "   ", wt)

# Minimum Hamming distance

d_min = np.min([w for w in weights if w != 0])

print("\nMinimum Hamming Distance:", d_min)

# -----------------------------

# Parity Check Matrix H = [I | P^T]

# -----------------------------

I_r = np.eye(col, dtype=int)

H = np.hstack((I_r, P.T))

print("\nParity Check Matrix (H):")

for r in H:

print(" ".join(map(str, r)))

H_T = H.T

# -----------------------------

# Receive Codeword

# -----------------------------

rc = list(map(int, input("\nEnter the received codeword : ").split()))

r = np.array(rc)

# Syndrome

S = np.mod(np.dot(r, H_T), 2)

print("Syndrome :", " ".join(map(str, S)))

# -----------------------------

# Error Detection & Correction

# -----------------------------

error = np.zeros(n, dtype=int)

for i in range(n):

if np.array_equal(H_T[i], S):

error[i] = 1

break

print("Error Vector :", " ".join(map(str, error)))

# Corrected codeword

corrected = np.mod(r + error, 2)

print("Corrected Codeword :", " ".join(map(str, corrected)))